PROTOTYPE DESIGN FOR MANKIND • AGRI-TECH INNOVATION

Project KRONOS

Smart In-Situ Soil Diagnostics, Spatial Nutrient Mapping & Hyperlocal AI Agronomy System

| Team: Team Kronos | Institution: Heritage / IIC (MHRD) | Hardware: ESP32-S3 + 7-in-1 Probe | AI Engine: Llama 3.3 70B (Groq) |
| --- | --- | --- | --- |

1) Problem Statement  [Word Count: 198 / 300 Max]

Smallholder agriculture in developing economies faces acute systemic inefficiencies that constrain crop yields and drive severe environmental degradation. Farmers currently operate with four structural handicaps:

1. Delayed Diagnostics: Centralized laboratory soil tests take 2 to 4 weeks, returning long after critical crop sowing and nutrient application windows have passed.
2. Raw Inactionable Data: Commercial IoT sensors simply dump raw electrical metrics (EC in uS/cm, raw moisture %) without translating them into kilograms of fertilizer needed.
3. Blanket Over-Fertilization: Uniform chemical application across uneven fields wastes 30–45% of fertilizers, accelerating groundwater contamination and soil acidification.
4. Late Disease Detection: Foliar pathologies are identified only after macroscopic collapse, causing irreversible yield losses of up to 40%.

Smallholders urgently need an affordable, in-situ diagnostic device that closes the loop between raw soil chemistry, weather forecasting, and clear, localized agricultural action.

2) Designed Solution to the Problem  [Word Count: 228 / 300 Max]

Project KRONOS is an integrated precision agronomy ecosystem that links portable edge hardware, cloud time-series ingestion, and deterministic multi-modal AI advisory:

• Edge Hardware Diagnostic Unit: A ruggedized field device featuring an ESP32-S3 microcontroller and an industrial 7-in-1 RS-485 Modbus probe. It measures Nitrogen (N), Phosphorus (P), Potassium (K), pH, Electrical Conductivity (EC), Moisture, and Temperature simultaneously in 15 seconds.
• Continuous Ingestion & Storage: Data streams to InfluxDB Cloud at 1-second intervals, materialized into daily Parquet and CSV structures for immediate and historical query access.
• Deterministic Agronomy Core: Executes 3-point rolling calibrations, Indian alluvial soil-specific NPK banding, and automated soil texture classification (Sandy/Clayey/Loamy) based on moisture dry-down slopes (dh/dt).
• Weather-Fused Generative AI: Merges live soil status with Open-Meteo 14-day forecasts and ECMWF SEAS5 seasonal outlooks. The payload prompts Llama 3.3 70B (via Groq) to generate exact, crop-specific fertilizer plans (kg/acre in split doses), irrigation advice, and disease risk alerts.
• Spatial Mapping & Vision: Enables single-probe temporal field walks to construct geostatistical nutrient/waterlogging heatmaps (Ordinary Kriging), alongside foliar disease scanning via Computer Vision.

3) Process & Pipeline Architecture  [End-to-End System Pipeline]

| Stage 1: Field Edge Hardware | ESP32-S3 reads 7-in-1 Modbus probe (N, P, K, pH, EC, Moisture, Temp) + NEO-M8N GPS every 1s. Local OLED display provides instant on-device feedback. |
| --- | --- |
| Stage 2: Cloud Ingestion Stream | Data streams via Wi-Fi/Cellular to InfluxDB Cloud. Python background worker converts stream into daily partitioned Parquet data lake and smoothed CSVs. |
| Stage 3: Analytical & AI Brain | Deterministic soil science classifies soil type and calculates NPK deficit bands. Payload fuses with Open-Meteo 14-day forecast and Llama 3.3 70B (Groq) for precision dosing. |
| Stage 4: Farmer Action Interface | Web app & PWA delivers exact fertilizer prescriptions (kg/acre in split DAS), Ordinary Kriging field heatmaps, waterlogging alerts, and leaf disease diagnostics. |

4) Core USP & Key Features of the Solution  [USP & Innovation Matrix]

⭐ Main Unique Selling Proposition (USP):
"Closing the Loop from Soil Chemistry to Field-Ready Prescriptions"
Unlike standard IoT soil probes that merely display raw, confusing numbers, KRONOS is an intelligent digital agronomist. It synthesizes 15-second in-situ 7-parameter soil diagnostics with 14-day hyperlocal weather forecasts to deliver exact metric fertilizer doses (kg/acre in split timings), single-probe nutrient heatmaps, and foliar disease cures in vernacular languages (Bengali, Hindi, English)—at less than 1/10th the cost of commercial lab setups.

Key Solution Features:

15-Second In-Situ Testing: Simultaneously captures N, P, K, pH, EC, moisture, and temperature in 15 seconds without laboratory wait times or chemical reagents.

Metric-Precision Fertilizer Prescriptions: Translates soil deficiencies into exact chemical and organic application rates (e.g., Urea @ 35 kg/acre in 2 split doses) tailored to crop stages.

Single-Probe Spatial Field Mapping: Leverages GPS-tagged temporal walks with a single hardware probe to construct high-resolution Ordinary Kriging nutrient and soil health heatmaps.

Waterlogging & Salinity Early Warning: Identifies standing water pooling versus dry zones by correlating EC and moisture dry-down slopes, preventing root asphyxiation.

14-Day Weather-Synchronized Advisory: Integrates rain probability and temperature forecasts to avoid fertilizing before heavy downpours and schedule pre-dawn heat irrigations.

Vision AI Leaf Disease Diagnostics: Identifies foliar pathogens from smartphone photos and cross-references them with soil moisture to assess disease escalation risks.

Multilingual Vernacular Interface: Delivers plain-language, emoji-rich agricultural advisories and conversational guidance in Bengali, Hindi, and English for smallholder accessibility.

5) Snapshots of Implementation  [2 to 4 Images]

| Figure 1: Hardware Prototype Node [ INSERT IMAGE: ESP32-S3 Edge Node, 7-in-1 Probe, OLED & Enclosure ] Technical Caption: ESP32-S3 edge node with RS-485 Modbus transceiver, 7-in-1 stainless probe, OLED display, and Li-Po power unit. | Figure 2: Real-Time Precision Dashboard [ INSERT IMAGE: Web Dashboard & NPK Live Status Cards ] Technical Caption: Web platform displaying live calibrated NPK status, soil classification, 14-day weather timeline, and Groq AI recommendations. |
| --- | --- |
| Figure 3: Spatial Nutrient & Moisture Heatmap [ INSERT IMAGE: Leaflet.js Kriging Interpolation Field Heatmap ] Technical Caption: Ordinary Kriging geostatistical interpolation map showing zone-wise Nitrogen levels and waterlogging accumulation. | Figure 4: Foliar Disease AI Diagnostic [ INSERT IMAGE: Leaf Scan Camera UI & AI Prescription Result ] Technical Caption: Computer vision module identifying leaf blight with chemical/organic treatment protocols and soil condition correlation. |

6) Impact to Society & Sustainable Model  [Word Count: 196 / 300 Max]

Project KRONOS directly advances United Nations Sustainable Development Goals (SDG 2: Zero Hunger, SDG 12: Responsible Production, and SDG 15: Life on Land):

1. Economic Prosperity for Smallholders: By replacing blind blanket fertilization with Variable Rate Application (VRA), farmers reduce chemical input costs by 20% to 35% while boosting crop yields by 15% to 25% through optimized macronutrient balance.
2. Environmental Rejuvenation: Monitoring in-situ pH and EC prevents secondary soil salinization and toxic nitrate runoff into village water supplies, promoting regenerative agricultural practices.
3. Climate Resilience: Fusing irrigation schedules with 14-day rain forecasts mitigates drought stress and prevents crop drowning, conserving millions of liters of freshwater.
4. Democratization of Agri-Tech: Commercial precision systems cost upwards of $3,000. KRONOS packages industrial-grade sensing into a sub-$65 BOM with free cloud intelligence, putting digital agronomists into smallholder pockets.

Sustainable Community Deployment Model:
• FPO Cooperative Hubs: Farmer Producer Organizations and Village Panchayats co-own 2–3 hardware nodes shared across 100+ local farmers.
• Freemium Digital App: Free real-time diagnostics + advisory; optional Rs 99/mo tier for exportable PDF audits & multi-field analytics.
• Bio-Input Marketplace: Direct referral integration with localized bio-fertilizer and micronutrient vendors for verified products.